import pandas as pd

import numpy as np

These are both used to import packages into the environment.

cubic_yard_ft3 = 27 #cubic feet in one cubic yard

unit_weight_water = 62.4 #lb/ft^3 (unit weight of water)

These are conversions to make cubic feet into one cubic yard and to get the unit weight of water into lb/ft^3.

print(f"Defined constant: 1 cubic yard = {cubic_yard_ft3} cubic feet")

print(f"Unit weight of water = {unit_weight_water} lb per cubic feet")

This function will print each part and its designated values, but will say Defined constant: 1 cubic yard = 27 cubic feet and the same style for the other line.

def calculate_water_weight_Q(cement_A, fly_ash_B, silica_fume_C, other_SCM_D, wc_ratio_E):

Calculating the water weight relies on the inputs made after the def calculate water weight.

total_cementitious = cement_A + fly_ash_B + silica_fume_C + other_SCM_D

water_weight_Q = total_cementitious * wc_ratio_E

return water_weight_Q

The formula for calculating water weight and how we get the code to correctly calculate that weight.

def calculate_volume_R(cement_A, sg_cement_J):

volume_R = cement_A / (sg_cement_J * unit_weight_water)

return volume_R

This is used to find the volume of cement, and the other three volumes (fly ash, silica fume, and other SCM) use this same calculation, but with their respective specific gravities and weight.

def calculate_volume_V(air_content_F):

volume_V = (air_content_F/100) * cubic_yard_ft3

return volume_V

Used to find the volume of air using the air content percentage.

def calculate_volume_W(water_weight_Q):

return water_weight_Q / unit_weight_water

Used to find the volume of water using the unit weight of water constant and the weight of water.

def calculate_total_aggregate_volume_X(volume_R, volume_S, volume_T, volume_U, volume_V, volume_W):

This is used to find the total aggregate volume using all the volumes calculated before.

volume_X = cubic_yard_ft3 - volume_R - volume_S - volume_T - volume_U - volume_V - volume_W

return volume_X

The formula used to find the total aggregate volume and the steps needed to find it.

def calculate_fine_aggregate_Y(percent_fine_G, sg_fine_N, volume_X):

weight_Y = unit_weight_water * (percent_fine_G / 100) * sg_fine_N * volume_X

return weight_Y

This is the calculation to find the fine aggregate needed using the specific gravity, total volume, percent fine aggregate, and the unit weight of water. This process can be replicated for coarse aggregate and other aggregates using their values for each input.

# Project metadata

project_number = int(input("Enter project number: "))

class_of_concrete = input("Enter class of concrete (e.g., 47B, 47BR): ")

This will give the user a prompt to input the project number and class of concrete they desire.

# Cementitious material inputs (lb per cubic yard)

cement_A = float(input("Enter cement weight A (lb per cubic yard): "))

fly_ash_B = float(input("Enter fly ash weight B (lb per cubic yard): "))

silica_fume_C = float(input("Enter silica fume weight C (lb per cubic yard): "))

other_SCM_D = float(input("Enter other SCM weight D (lb per cubic yard): "))

water_cement_ratio_E = float(input("Enter target water-cement ratio E: "))

air_content_F = float(input("Enter target air content F (%): "))

These are all the cementitious material inputs that are needed for the calculations.

# Aggregate proportions (%)

percent_fine_G = float(input("Enter percent fine aggregate G (%): "))

percent_coarse_H = float(input("Enter percent coarse aggregate H (%): "))

percent_other_I = float(input("Enter percent other aggregate I (%): "))

These are the percent aggregate proportions desired for the user to input.

# Specific gravities

sg_cement_J = float(input("Enter specific gravity of cement J: "))

sg_fly_ash_K = float(input("Enter specific gravity of fly ash K: "))

sg_silica_fume_L = float(input("Enter specific gravity of silica fume L: "))

sg_other_SCM_M = float(input("Enter specific gravity of other SCM M: "))

sg_fine_N = float(input("Enter specific gravity of fine aggregate N: "))

sg_coarse_O = float(input("Enter specific gravity of coarse aggregate O: "))

sg_other_P = float(input("Enter specific gravity of other aggregate P: "))

These are the specific gravities needed for the user to input.

# Calculations

Q = calculate_water_weight_Q(

cement_A, fly_ash_B, silica_fume_C, other_SCM_D, water_cement_ratio_E

)

R = calculate_volume_R(cement_A, sg_cement_J)

S = calculate_volume_S(fly_ash_B, sg_fly_ash_K)

T = calculate_volume_T(silica_fume_C, sg_silica_fume_L)

U = calculate_volume_U(other_SCM_D, sg_other_SCM_M)

V = calculate_volume_V(air_content_F)

W = calculate_volume_W(Q)

X = calculate_total_aggregate_volume_X(R, S, T, U, V, W)

Y = calculate_fine_aggregate_Y(percent_fine_G, sg_fine_N, X)

Z = calculate_coarse_aggregate_Z(percent_coarse_H, sg_coarse_O, X)

AA = calculate_other_aggregate_AA(percent_other_I, sg_other_P, X)

These are all the calculations we did above, and now we reduce the name of the calculation to one letter to make it an easier input and something to call the code back to.

# Output: Final Weight Chart

print("\n---------------------------------------------")

print(" NDOT Concrete Mix Design – Weight Summary")

print("         (1 Cubic Yard of Concrete)")

print("---------------------------------------------")

print(f"Project Number:       {project_number}")

print(f"Class of Concrete:    {class_of_concrete}")

print("---------------------------------------------")

print(f"Cement (A):            {cement_A:8.1f} lb")

print(f"Fly Ash (B):           {fly_ash_B:8.1f} lb")

print(f"Silica Fume (C):       {silica_fume_C:8.1f} lb")

print(f"Other SCM (D):         {other_SCM_D:8.1f} lb")

print("---------------------------------------------")

print(f"Fine Aggregate (Y):    {Y:8.1f} lb")

print(f"Coarse Aggregate (Z):  {Z:8.1f} lb")

print(f"Other Aggregate (AA):  {AA:8.1f} lb")

print("---------------------------------------------")

print(f"Water (Q):             {Q:8.1f} lb")

print("---------------------------------------------")

print("End of Mix Design Summary")

This is the output table and has 8.1f with 8 being the minimum field width of 8 characters, .1 is to display one digit after the decimal, and the f is to format the value as a floating-point number. Then the rest of the code is just to print it into a valid format as described in the code layout.